|  | Advantages | Disadvantages |
| --- | --- | --- |
| 2L | - Lower conduction loss - High efficiency at lower switching frequency. - Simple to implement and control. - 6 switches, 6 gate drivers, of which 3 are isolated. - Cheap. - Reliable. | - Higher switching loss. - Low efficiency at higher switching frequency. - Higher harmonic distortion and dv/dt. - Higher blocking voltage (VDC). |
| 2L-FB Dual inverters | - Lower dv/dt. - Higher utilization of DC-link. | - Higher conduction and switching loss. - 12 switches, 12 gate drivers, of which 6 are isolated. - Expensive. - Higher blocking voltage (VDC). |
| 3L T-type | - Moderate conduction loss. - Lower switching loss. - High efficiency at medium switching frequency. - Lower harmonic distortion and dv/dt. - 12 switches, 12 gate drivers, of which 6 are isolated - Half of them lower blocking voltage (VDC/2) | - Low efficiency at higher switching frequency. - Complex to implement and control. - Expensive. - Half of them higher blocking voltage (VDC) - Reduced reliability. - Active DC-link balancing is required. |
| 3L NPC | - Lower switching loss. - High efficiency at higher switching frequency. - Lower harmonic distortion and dv/dt. - 12 switches-6 diodes, 12 gate drivers, of which 9 are isolated - Lower blocking voltage (VDC/2) | - Higher conduction loss. - Low efficiency at lower switching frequency. - Complex to implement and control. - Expensive - Reduced reliability. - Active DC-link balancing is required. |
| 3L ANPC | - Lower switching loss. - High efficiency at higher switching frequency. - Lower harmonic distortion and dv/dt. - 18 switches, 18 gate drivers, of which 15 are isolated - Lower blocking voltage (VDC/2) - Gives more control flexibility. | - Higher conduction loss. - Low efficiency at lower switching frequency. - Complex to implement and control. - Expensive - Reduced reliability. - Active DC-link balancing is required. |
| SPB / MMD | - Higher efficiency and lower losses by adjusting the series/parallel modules. - Providing high frequency. - High power density thanks to advantages of mechanical integration and cooling system. - The lowest harmonic distortion and dv/dt. - Reliable thanks to being modular and providing redundancy. - Easy to scale. | - Number of modules x 6 gate drivers, of which x3 are isolated. - Expensive. - Complex. |
| MMC | - Higher efficiency. - Providing high frequency. - The lowest harmonic distortion and dv/dt. - Modularity. | - Number of modules x 2 (or 4) gate drivers, of which x 1 (or) are isolated. - Expensive. - Complex to control. - Lower power density due to branch inductors. |