Integrated Auxiliary Power Supply Using Switching Harmonics in an Open-End Winding Traction Drive

Abstract

Electric vehicle powertrains conventionally employ a dedicated high-voltage to low-voltage DC/DC converter to supply auxiliary loads such as control electronics, cooling systems, and low-voltage buses. Although the auxiliary power demand is small compared with the traction power, the additional converter requires semiconductor devices, magnetic components, cooling, protection, and packaging. This paper proposes an integrated auxiliary power supply concept for an open-end winding traction drive. The modulation redundancy provided by the two traction inverters is utilized to control the high-frequency differential voltage across the machine winding while maintaining the required fundamental voltage for torque production. A parallel resonant branch, tuned close to a selected switching-frequency component, presents high impedance at the machine fundamental frequency and low impedance at the targeted harmonic. The resulting high-frequency power is rectified to supply a DC auxiliary load. The concept is evaluated in terms of obtainable auxiliary power, interaction with traction operation, and additional converter losses.

Introduction

Battery electric vehicle (BEV) drivetrains contain several power-electronic converters with different power and voltage ratings. The highest-power converters are traction inverters that energize and control the electric machine from the high-voltage battery. In addition, there are several auxiliary converters which are commonly utilized to supply low-voltage loads such as lighting, electronic control units, pumps, cooling systems, etc.

These auxiliary converters require considerably smaller power compared to traction inverters.

However, they usually need a high-voltage high-step-down converter where the design may be tricky regarding the selection of semiconductor devices, magnetic components, gate drivers, and cooling systems.

Therefore, a system-level perspective shows that these converters increase the component count, volume, weight, and cost.

One smart idea here to mitigate these is to combine some of the converters to utilize the components more. For example, it is possible to put a pre-scaling stage, then a regulation stage can drive multiple ports [1].

At the same time, the traction inverter continuously generates high-frequency voltage components as a consequence of pulse-width modulation (PWM). These components are normally treated as undesirable harmonics because they contribute to current ripple, additional losses, and electromagnetic interference. However, they also constitute a controllable high-frequency excitation that can potentially be used for another power-conversion function.

An open-end winding (OEW) machine provides additional flexibility for controlling these components because both terminals of each phase winding are supplied by independent inverter legs. Different switching-state combinations can synthesize the same fundamental machine voltage while producing different high-frequency differential voltages. This paper utilizes that modulation redundancy to generate auxiliary power from the traction drive. A resonant branch is connected in parallel with the machine winding and tuned near a selected switching-frequency component. The branch is designed to have high impedance at the machine fundamental frequency and low impedance at the selected harmonic.

The main contribution is an integrated auxiliary power supply concept in which the traction inverters simultaneously synthesize the fundamental machine voltage and a controllable high-frequency excitation for a resonant auxiliary branch. The approach therefore offers a possible method for reducing or simplifying the conventional dedicated high-voltage auxiliary converter.

PROPOSED SYSTEM CONFIGURATION

The proposed system is based on an OEW electric machine supplied by two voltage-source inverters. Considering phase A, the instantaneous winding voltage is determined by the difference between the two inverter terminal voltages:

| VAA (t) = VA1(t) - VA2 (t) | (1) |
| --- | --- |

The winding voltage can be separated conceptually into a fundamental component responsible for machine operation and high-frequency components introduced by PWM:

| VAA(t) = VAA,1(t) + VAA,h(t) | (2) |
| --- | --- |

For normal traction operation, the fundamental component is selected according to the required machine torque and flux. The high-frequency component, however, depends strongly on the switching sequence and relative modulation of the two inverters. The proposed method uses this degree of freedom to control the high-frequency differential voltage without intentionally changing the commanded fundamental voltage.

Figure 1: Proposed open-end winding traction drive with parallel resonant auxiliary power extraction branch.

A high-frequency resonant branch is connected in parallel with the machine phase, followed by a rectifier and a DC-side filter or auxiliary load. Its impedance is selected such that the branch draws only limited current at the fundamental frequency while providing a low-impedance path around the selected switching-frequency component.

| ¦Zaux(f1)¦ >> ¦Zaux(fs)¦ | (3) |
| --- | --- |

For a simplified LC resonant network, the nominal resonant frequency is

| fr = 1 / (2π√ (Laux Caux)) | (4) |
| --- | --- |

where Laux and Caux are selected so that the resonance is close to the targeted switching-frequency component. The resulting high-frequency current is subsequently rectified to obtain a DC auxiliary output.

MODULATION-BASED AUXILIARY POWER CONTROL

The key feature of the proposed concept is that the auxiliary power is regulated through the modulation of the traction inverters rather than by an additional active high-voltage conversion stage. For the same commanded fundamental winding voltage, the relative switching patterns of the two inverters can be varied to alter the differential high-frequency voltage.

One possible control variable is the carrier phase displacement between inverter 1 and inverter 2. If the two carrier signals have a relative phase displacement Δφ, the resulting high-frequency voltage across the winding changes with Δφ, while the fundamental voltage reference can remain unchanged. The selected harmonic can therefore be represented as

| Vh = f(Δφ, m, fs, Vdc) | (5) |
| --- | --- |

where m is the modulation index, fs is the switching frequency, and Vdc is the inverter DC-link voltage. Carrier phase shift is not the only possible degree of freedom; switching-state selection, PWM timing, and controlled zero-sequence components may also be used to shape the available high-frequency voltage.

The proposed approach therefore separates the two functions spectrally. The fundamental component is used for electromechanical energy conversion, whereas a selected switching-frequency component is intentionally used to excite the auxiliary resonant branch.

RESONANT AUXILIARY POWER EXTRACTION AND DRIVE INTERACTION

The resonant branch behaves as a frequency-selective load. At the machine fundamental frequency, its high impedance minimizes the auxiliary fundamental current. Around the selected resonant frequency, the branch impedance decreases and the switching-frequency voltage produces a substantially larger high-frequency current.

| Iaux,h = Vh / Zaux(f_h) | (6) |
| --- | --- |

The corresponding active power transferred to the equivalent auxiliary load can be approximated from the RMS value of the selected harmonic voltage and the equivalent resistance seen through the resonant network and rectifier:

| Paux ≈ Vh,rms² / Req | (7) |
| --- | --- |

This relation illustrates the basic regulation principle: varying the controllable harmonic voltage changes the auxiliary power while the fundamental machine-voltage command is maintained.

The switching harmonics should not be interpreted as a separate source of free or previously lost energy. The auxiliary energy ultimately originates from the DC source and is transferred through the existing traction inverter switching stage. The high-frequency voltage components provide the controlled carrier for this additional power-transfer path.

Although the expected auxiliary power is much smaller than the traction rating, the additional high-frequency current can increase semiconductor conduction and switching losses and may also contribute to machine and resonant-component losses. Therefore, the feasibility of the concept must be assessed using the overall system power balance.

| ηaux,int = Paux,out / (Paux,out + ΔPinv + Pres + Prect) | (8) |
| --- | --- |

where ΔPinv is the incremental traction-inverter loss caused by auxiliary extraction, Pres represents resonant-network losses, and Prect represents rectifier losses. This metric provides a direct basis for comparison with a conventional dedicated auxiliary converter.

Conclusions and Future Work

This paper proposes an integrated auxiliary power supply for an open-end winding traction drive using modulation-controlled switching-frequency excitation. The two traction inverters synthesize the required fundamental machine voltage while their relative modulation is used to control the high-frequency differential voltage across the winding. A parallel resonant branch selectively extracts this high-frequency power and a rectifier converts it to a DC auxiliary supply.

Compared with a conventional EV architecture, the proposed approach uses the existing traction switching stage as part of the auxiliary power-conversion process, potentially reducing dedicated semiconductor hardware and converter volume.

Future work will quantify auxiliary power capability, resonant-network stresses, additional losses, and operating-point dependence, followed by simulation validation.
References

[1]	A. Jones, B. Smith, and C. Maxwell, “Title of their journal paper,” IEEE Transactions on Power Electronics, vol. 17, no. 1, pp. 45-55, Jan. 1995.

[2]	A. Jones, B. Smith, and C. Maxwell, “Title of their conference paper,” in IEEE 1997 Applied Power Electronics Conference, 1997, pp. 455-452.

[3]	A. Jones, Title of book, Publisher’s Name, City, 2004.